Universal Cabinet Lock User Manual

1----------------------------------------------------------------------------------------Package

2----------------------------------------------------------------------------------------Installation Guide

3----------------------------------------------------------------------------------------Program Main Card Steps

4-----------------------------------------------------------------------------Low Voltage Alarm

- 1, Package:

| - Sticker | - Lock Body | - Latch |
| --- | --- | --- |
| - Main Card | - Screws | - Power Supplier(Additional Purchase) |

- 2, Installation Guide

- 3, Program Main Card Steps

- 3.1 Initialization button is beside battery box, inside the small hole. Use a toothpick or slight stick to press the button.

- 3.2 Release toothpick or slight stick after a “beep” sound, lock will be single card mode.

- Release toothpick or slight stick after 2 “beep” sounds, lock will be double card mode.

- * Single card mode means lock can be opened by main card or key card

- * Double card mode means lock just can be opened by reading main card(first) and key card.

- 3.3 After initialized, the first and second card read on lock will be main card.

- * If one main card lost, read another main card 3 times, after a long “ beep” sound, read a new card, this card will be the second main card, at the same time, the lost main card failed.

- 4, Low Voltage Alarm

- 4.1 When voltage is low, there is a 8 times “beep” sound after read key card, and then lock bolt will not extend, lock will be normal open mode. Please replace all batteries.

- * Read main card 9 times can close low voltage alarm function or active this function. The default mode is activated.