Password Sauna Lock

Public Mode Configuration: Main Password*1, Temporary Password*1, Main Fingerprint*1, Temporary Fingerprint*1

Password: 4~15 digits.

- 1, Factory Reset

Press button on back side of mortise by a thin stick for 5 times, after this step, the lock status will be:

Default Mode: Home Using, Single Open

Main Password: 12345678

Fingerprint: cleared

- 2, Change to Public Lock Mode

Main Password + Key + 7 + Key : Change to public lock mode, lock will be normal open;

- 3, Operations:

- A, Manage Users(Operate in Normal Open and Normal Close)

1, Add Main Fingerprint: the first fingerprint on lock will be the main fingerprint.

2, Revise Main Password

Previous Main Password + Key + 1 + New Main Password + Key + New Main Password + Key

B, Add Temporary Users(Operate in Normal Open)

1, Add Temporary Password (Single Open Mode)

New Temporary Password + Key + New Temporary Password + Key;    Lock Close

2, Add Temporary Fingerprint (Single Open Mode)

Put Fingerprint on lock; Lock Close

3, Add Temporary Password and Fingerprint (Double Open Mode):

New Temporary Password+ Key + New Temporary Password + Key and press fingerprint; Lock Close

Or

Press Fingerprint and New Temporary Password+ Key + New Temporary Password + Key; Lock Close

C, System Settings:

1, Home Using, Public Mode Switch to Each Other

Main Password + Key + 7 + Key

2, Single Open Mode Switch to Double Open Mode:

Main Password + Key + 6 + Key

3, Double Open Mode Switch to Single Open Mode:

Main Password + Key + 5 + Key

D, Opening

1, Single Open Mode

Password + Key;         Lock Open, Temporary Password and Fingerprint Invalid

Press Fingerprint;        Lock Open, Temporary Password and Fingerprint Invalid

2, Double Open Mode:

Main Password + Key (Or Main Fingerprint); Lock Open, Temporary Password and Fingerprint Invalid

User Password + Key (Or Fingerprint)+ Fingerprint (Or Password+key); Lock Open, Temporary Password and Card Invalid

Password ID Card Home Using Lock

- 1, Factory Reset

Press button on back side of mortise by a thin stick for 5 times, after this step, the lock status will be:

Password: 12345678

Default Mode: Home Using, Single Open

Capacity:

Main Password: 1

User Password: 200

Main Fingerprint: 1

User Fingerprint: 50

Password: 4~15 digits

- 2, Change to Home Using Mode

Main Password + Key + 7 + Key : Change to home using mode, lock close

- 3, Operations:

- A, Main Password and Main Fingerprint Management

1, Add Main Fingerprint:

Lock after factory reset, the first fingerprint will be the main fingerprint;

- 2, Revise Main Password

Previous Main Password + Key + 1 + New Main Password + Key + New Main Password + Key

B, Add User

1, Add User Password

Main Password + Key + 8 + NO.(001~200) + Key + User Password + Key + User Password + Key

2, Add User Fingerprint:

Main Fingerprint + NO.(01~50) + Key + New Fingerprint;

C, Delete User

1, Delete User Password:

Main Password + Key + 4 + No.(001~200) + Key + No.(001~200) + Key;

2, Delete User Fingerprint:

Main Fingerprint + 4 + No.(01~50) + Key + No.(01~50)+Key;

3, Delete All User Passwords:

Main Password + Key + 0 + Key

4, Delete All User Fingerprints:

Main Fingerprint + 0 + Key

D, System Settings:

1, Home Using Mode and Public Mode Changing:

Main Password + Key + 7 + Key

2, Single Open Change to Double Open Mode:

Main Password + Key + 6 + Key

3, Double Open Mode Change to Single Open Mode:

Main Password + Key + 5 + Key

E, Open:

1, Single Open:

Press Fingerprint;      Lock Open, Close After 5 Seconds

User Password + Key;  Lock Open, Close After 5 Seconds

In 5 seconds before lock close, password can be revised:

1 + New Password + Key + New Password + Key;

2, Double Open:

Main Password + Key (or Main Fingerprint); Lock Open, Close After 5 Seconds

User Password + Key (Or User Fingerprint) + User Fingerprint (Or User Password + Key); Lock Open, Close After 5 Seconds

Note: Double Open Mode do not support user password revising.

Note: Input wrong password over 7 times, password panel will be locked for 2 minutes.

- D, Other Functions

1, If password invalid, speaker have 3 times “ dee~ “ sound, blue backlight lights down.

2, If low voltage, there is a warning of low voltage, if voltage below the limit, user card can not open lock, only main card can open..

Special Note:

Lock support false password. For example: the right password is 123456, other numbers can add to the start or end of password, like 01234567890 or 78901234560

Features:

Working Voltage: 6.0V (4 AA Batteries)

Power: ＜300mA

Low Voltage Warning: 4.8V

Reading Distance: 0~10cm